Listing of IBM PC Diagnostic Error Codes

Jerry Schneider

Capital PC User Group

Washington, DC

All personal computer error codes for the Power On Self Test, General

Diagnostics, and Advanced Diagnostics consist of a device number followed by

two digits other than 00.  (The device number plus 00 indicates successful

completion of the test.)

This list is a compilation from various sources, including USENET's Info-IBMPC

Digest, IBM Technical Reference Manuals, and IBM Hardware, Maintenance and

Service manuals.

Code   Description

01x   Undetermined problem errors.

02x   Power supply errors.

1xx   System board errors.

101   System board error - Interrupt failure.

102   System board error - Timer failure.

103   System board error - Timer interrupt failure.

104   System board error - Protected mode failure.

105   System board error - Last 8042 command not accepted.

106   System board error - Converting logic test.

107   System board error - Hot NMI test.

108   System board error - Timer bus test.

109   Direct memory access test error.

121   Unexpected hardware interrupts occurred.

131   Cassette wrap test failed.

152

161   System Options Error-(Run SETUP) [Battery failure].

162   System options not set correctly-(Run SETUP).

163   Time and date not set-(Run SETUP).

164   Memory size error-(Run SETUP).

199   User-indicated configuration not correct.

2xx   Memory (RAM) errors.

201   Memory test failed.

202   Memory address error.

203   Memory address error.

3xx   Keyboard errors.

301   Keyboard did not respond to software reset correctly, or a stuck

key failure was detected.  If a stuck key was detected, the

scan code for the key is dis- played in hexadecimal.  For

example, the error code 49 301 indicates that key 73, the

PgUp key, has failed (49 hex = 73 decimal).

302   User-indicated error from the keyboard test, or AT keylock

is locked.

303   Keyboard or system unit error.

304   Keyboard or system unit error; CMOS does not match system.

4xx   Monochrome monitor errors.

401   Monochrome memory test, horizontal sync frequency test, or

video test failed.

408   User-indicated display attributes failure.

416   User-indicated character set failure.

424   User-indicated 80 X 25 mode failure.

432   Parallel port test failed (monochrome adapter).

5xx   Color monitor errors.

501   Color memory test failed, horizontal sync frequency test, or

video test failed.

508   User-indicated display attribute failure.

516   User-indicated character set failure.

524   User-indicated 80 X 25 mode failure.

532   User-indicated 40 X 25 mode failure.

540   User-indicated 320 X 200 graphics mode failure.

548   User-indicated 640 X 200 graphics mode failure.

6xx   Diskette drive errors.

601   Diskette power-on diagnostics test failed.

602   Diskette test failed; boot record is not valid.

606   Diskette verifysd function failed.

607   Write-protected diskette.

608   Bad command diskette status returned.

610   Diskette initialization failed.

611   Timeout - diskette status returned.

612   Bad NEC - diskette status returned.

613   Bad DMA - diskette status returned.

621   Bad seek - diskette status returned.

622   Bad CRC - diskette status returned.

623   Record not found - diskette status returned.

624   Bad address mark - diskette status returned.

625   Bad NEC seek - diskette status returned.

626   Diskette data compare error.

7xx   8087 or 80287 math coprocessor errors.

9xx   Parallel printer adapter errors.

901   Parallel printer adapter test failed.

10xx   Reserved for parallel printer adapter.

11xx   Asynchronous communications adapter errors.

1101   Asynchronous communications adapter test failed.

12xx   Alternate asynchronous communications adapter errors.

1201   Alternate asynchronous communications adapter test failed.

13xx   Game control adapter errors.

1301   Game control adapter test failed.

1302   Joystick test failed.

14xx   Printer errors.

1401   Printer test failed.

1404   Matrix printer failed.

15xx   Synchronous data link control (SDLC) communications adapter errors.

1510   8255 port B failure.

1511   8255 port A failure.

1512   8255 port C failure.

1513   8253 timer 1 did not reach terminal count.

1514   8253 timer 1 stuck on.

1515   8253 timer 0 did not reach terminal count.

1516   8253 timer 0 stuck on.

1517   8253 timer 2 did not reach terminal count.

1518   8253 timer 2 stuck on.

1519   8273 port B error.

1520   8273 port A error.

1521   8273 command/read timeout.

1522   Interrupt level 4 failure.

1523   Ring Indicate stuck on.

1524   Receive clock stuck on.

1525   Transmit clock stuck on.

1526   Test indicate stuck on.

1527   Ring indicate not on.

1528   Receive clock not on.

1529   Transmit clock not on.

1530   Test indicate not on.

1531   Data set ready not on.

1532   Carrier detect not on.

1533   Clear to send not on.

1534   Data set ready stuck on.

1536   Clear to send stuck on.

1537   Level 3 interrupt failure.

1538   Receive interrupt results error.

1539   Wrap data miscompare.

1540   DMA channel 1 error.

1541   DMA channel 1 error.

1542   Error in 8273 error checking or status reporting.

1547   Stray interrupt level 4.

1548   Stray interrupt level 3.

1549   Interrupt presentation sequence timeout.

16xx   Display emulation errors (327x, 5520, 525x).

17xx   Fixed disk errors.

1701   Fixed disk POST error.

1702   Fixed disk adapter error.

1703   Fixed disk drive error.

1704   Fixed disk adapter or drive error.

1780   Fixed disk 0 failure.

1781   Fixed disk 1 failure.

1782   Fixed disk controller failure.

1790   Fixed disk 0 error.

1791   Fixed disk 1 error.

18xx   I/O expansion unit errors.

1801   I/O expansion unit POST error.

1810   Enable/Disable failure.

1811   Extender card warp test failed (disabled).

1812   High order address lines failure (disabled).

1813   Wait state failure (disabled).

1814   Enable/Disable could not be set on.

1815   Wait state failure (disabled).

1816   Extender card warp test failed (enabled).

1817   High order address lines failure (enabled).

1818   Disable not functioning.

1819   Wait request switch not set correctly.

1820   Receiver card wrap test failure.

1821   Receiver high order address lines failure.

19xx   3270 PC attachment card errors.

20xx   Binary synchronous communications (BSC) adapter errors.

2010   8255 port A failure.

2011   8255 port B failure.

2012   8255 port C failure.

2013   8253 timer 1 did not reach terminal count.

2014   8253 timer 1 stuck on.

2016   8253 timer 2 did not reach terminal count, or timer 2 stuck on.

2017   8251 Data set ready failed to come on.

2018   8251 Clear to send not sensed.

2019   8251 Data set ready stuck on.

2020   8251 Clear to send stuck on.

2021   8251 hardware reset failed.

2022   8251 software reset failed.

2023   8251 software "error reset" failed.

2024   8251 transmit ready did not come on.

2025   8251 receive ready did not come on.

2026   8251 could not force "overrun" error status.

2027   Interrupt failure - no timer interrupt.

2028   Interrupt failure - transmit, replace card or planar.

2029   Interrupt failure - transmit, replace card.

2030   Interrupt failure - receive, replace card or planar.

2031   Interrupt failure - receive, replace card.

2033   Ring indicate stuck on.

2034   Receive clock stuck on.

2035   Transmit clock stuck on.

2036   Test indicate stuck on.

2037   Ring indicate stuck on.

2038   Receive clock not on.

2039   Transmit clock not on.

2040   Test indicate not on.

2041   Data set ready not on.

2042   Carrier detect not on.

2043   Clear to send not on.

2044   Data set ready stuck on.

2045   Carrier detect stuck on.

2046   Clear to send stuck on.

2047   Unexpected transmit interrupt.

2048   Unexpected receive interrupt.

2049   Transmit data did not equal receive data.

2050   8251 detected overrun error.

2051   Lost data set ready during data wrap.

2052   Receive timeout during data wrap.

21xx   Alternate binary synchronous communications adapter errors.

2110   8255 port A failure.

2111   8255 port B failure.

2112   8255 port C failure.

2113   8253 timer 1 did not reach terminal count.

2114   8253 timer 1 stuck on.

2115   8253 timer 2 did not reach terminal count, or timer   2 stuck on.

2116   8251 Data set ready failed to come on.

2117   8251 Clear to send not sensed.

2118   8251 Data set ready stuck on.

2119   8251 Clear to send stuck on.

2120   8251 hardware reset failed.

2121   8251 software reset failed.

2122   8251 software "error reset" failed.

2123   8251 transmit ready did not come on.

2124   8251 receive ready did not come on.

2125   8251 could not force "overrun" error status.

2126   Interrupt failure - no timer interrupt.

2128   Interrupt failure - transmit, replace card or planar.

2129   Interrupt failure - transmit, replace card.

2130   Interrupt failure - receive, replace card or planar.

2131   Interrupt failure - receive, replace card.

2133   Ring indicate stuck on.

2134   Receive clock stuck on.

2135   Transmit clock stuck on.

2136   Test indicate stuck on.

2137   Ring indicate stuck on.

2138   Receive clock not on.

2139   Transmit clock not on.

2140   Test indicate not on.

2141   Data set ready not on.

2142   Carrier detect not on.

2143   Clear to send not on.

2144   Data set ready stuck on.

2145   Carrier detect stuck on.

2146   Clear to send stuck on.

2147   Unexpected transmit interrupt.

2148   Unexpected receive interrupt.

2149   Transmit data did not equal receive data.

2150   8251 detected overrun error.

2151   Lost data set ready during data wrap.

2152   Receive timeout during data wrap.

22xx   Cluster adapter errors.

24xx   Enhanced graphics adapter errors.

29xx   Color matrix printer errors.

2901

2902

2904

33xx   Compact printer errors.

####